Lab 3 (Task A)

VHDL code for ROM

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity ROM is

Port ( addr : in  UNSIGNED (3 downto 0);

fib_out : out  STD_LOGIC_VECTOR (7 downto 0));

end ROM;

architecture Behavioral of ROM is

type ROM_Array is array (0 to 15) of std_logic_vector(7 downto 0);

constant Fibonacci: ROM_Array:=(            --Assignment of Fibonacci numbers to corresponding array address in ascending order

0  => "00000000",   --0

1  => "00000001",   --1

2  => "00000001",   --1

3  => "00000010",   --2

4  => "00000011",   --3

5  => "00000101",   --5

6  => "00001000",   --8

7  => "00001101",   --13

8  => "00010101",   --21

9  => "00100010",   --34

10 => "00110111",   --55

11 => "01011001",   --89

12 => "10010000",   --144

13 => "11101001",   --233

others => "00000000");  --For other remaining values = 0

begin

fib_out <= Fibonacci(to_integer(addr)); --Output corresponding Fibonacci number to the address called,

end Behavioral;

VHDL code for counter

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity counter is

Port ( clk : in  STD_LOGIC;                         --clock input

rst : in  STD_LOGIC;                         --reset signal

count : in  STD_LOGIC;                       --count enable

addr_out : out  UNSIGNED (3 downto 0));  --counter output (address)

end counter;

architecture Behavioral of counter is

--internal counter signal

signal count_signal : UNSIGNED (3 downto 0);

begin

--count process

count_proc: process (clk)

begin

--on rising edge of clock

if rising_edge(clk) then

--if reset is high

if rst = '1' then

--set count to zero

count_signal <= (others => '0');

else

--if count is high

if count = '1' then

--if the counter has reached the end of the address range

if count_signal = "1101" then

--counter should loop around to 0

count_signal <= (others => '0');

else

--counter should increment by one

count_signal <= count_signal + 1;

end if;

end if;

end if;

end if;

end process count_proc;

--connect the output

addr_out <= count_signal;

end Behavioral;

VHDL code for top level system

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity fibonacci_system is

Port ( clk : in  STD_LOGIC;

rst : in  STD_LOGIC;

count : in  STD_LOGIC;

fib_out : out  STD_LOGIC_VECTOR (7 downto 0));

end fibonacci_system;

architecture Behavioral of fibonacci_system is

signal deb_rst: STD_LOGIC;

signal deb_count: STD_LOGIC;

signal fib_addr: UNSIGNED (3 downto 0);

--uses debouncer component

COMPONENT debouncer

PORT(

clk : IN  std_logic;

Sig : IN  std_logic;

Deb_Sig : OUT  std_logic

);

END COMPONENT;

--uses counter component

COMPONENT counter

PORT(

clk : IN std_logic;

rst : IN std_logic;

count : IN std_logic;

addr_out : OUT UNSIGNED(3 downto 0)

);

END COMPONENT;

--uses ROM component

COMPONENT ROM

PORT(

addr : IN UNSIGNED(3 downto 0);

fib_out : OUT std_logic_vector(7 downto 0)

);

END COMPONENT;

begin

--debouncer for the reset input button

debouncerRst: debouncer port map(

Sig => rst,

Deb_Sig => deb_rst,

clk => clk

);

--debouncer for the count input button

debouncerCount: debouncer port map(

Sig => count,

Deb_Sig => deb_count,

clk => clk

);

--Counter (0-13) for generating addresses for rom

counter0: counter port map(

rst => deb_rst,

count => deb_count,

addr_out => fib_addr,

clk => clk

);

--fibonacci rom, takes an address and provides the fibonacci number

fib_rom: ROM port map(

addr => fib_addr,

fib_out => fib_out

);

end Behavioral;

Testbench Code

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

USE ieee.numeric_std.ALL;

ENTITY fibonacci_system_tb IS

END fibonacci_system_tb;

ARCHITECTURE behavior OF fibonacci_system_tb IS

-- Component Declaration for the Unit Under Test (UUT)

COMPONENT fibonacci_system

PORT(

clk : IN  std_logic;

rst : IN  std_logic;

count : IN  std_logic;

fib_out : OUT  std_logic_vector(7 downto 0)

);

END COMPONENT;

--Inputs

signal clk : std_logic := '0';

signal rst : std_logic := '0';

signal count : std_logic := '0';

--Outputs

signal fib_out : std_logic_vector(7 downto 0);

-- Clock period definitions

constant clk_period : time := 10 ns;

BEGIN

-- Instantiate the Unit Under Test (UUT)

uut: fibonacci_system PORT MAP (

clk => clk,

rst => rst,

count => count,

fib_out => fib_out

);

-- Clock process definitions

clk_process :process

begin

clk <= '0';

wait for clk_period/2;

clk <= '1';

wait for clk_period/2;

end process;

-- Stimulus process

stim_proc: process

begin

-- hold reset state for 100 ns.

wait for 100 ns;

--Reset the system (wait for 2 clock periods because of debouncer)

rst <= '1';

count <= '0';

wait for clk_period*2;

rst <= '0';

wait for clk_period*2;

--Advance by one count

count <= '1';

wait for clk_period*3;

count <= '0';

wait for clk_period*3;

--test second reset

rst <= '1';

wait for clk_period*2;

rst <= '0';

--keep counting forever to loop through the numbers

COUNT_LOOP : loop

count <= '1';

wait for clk_period*3;

count <= '0';

wait for clk_period*3;

end loop COUNT_LOOP;

wait;

end process;

END;

UCF file

# PlanAhead Generated physical constraints

NET "clk" LOC = L15;

NET "count" LOC = F5;

NET "rst" LOC = T15;

NET "fib_out[0]" LOC = U18;

NET "fib_out[1]" LOC = M14;

NET "fib_out[2]" LOC = N14;

NET "fib_out[3]" LOC = L14;

NET "fib_out[4]" LOC = M13;

NET "fib_out[5]" LOC = D4;

NET "fib_out[6]" LOC = P16;

NET "fib_out[7]" LOC = N12;

Simulation

HDL synthesis report

=========================================================================

*                           HDL Synthesis                               *

=========================================================================

Synthesizing Unit <fibonacci_system>.

Related source file is "C:\Users\*******\Digital_Electronics_Labs\Lab3_TaskA\fibonacci_system.vhd".

Summary:

no macro.

Unit <fibonacci_system> synthesized.

Synthesizing Unit <Debouncer>.

Related source file is "C:\Users\*******\Digital_Electronics_Labs\Lab3_TaskA\debouncer.vhd".

Found 1-bit register for signal <Q1>.

Found 1-bit register for signal <Q2>.

Found 1-bit register for signal <Q0>.

Summary:

inferred   3 D-type flip-flop(s).

Unit <Debouncer> synthesized.

Synthesizing Unit <counter>.

Related source file is "C:\Users\*******\Digital_Electronics_Labs\Lab3_TaskA\counter.vhd".

Found 4-bit register for signal <count_signal>.

Found 4-bit adder for signal <count_signal[3]_GND_6_o_add_1_OUT> created at line 1241.

Summary:

inferred   1 Adder/Subtractor(s).

inferred   4 D-type flip-flop(s).

Unit <counter> synthesized.

Synthesizing Unit <ROM>.

Related source file is "C:\Users\*******\Digital_Electronics_Labs\Lab3_TaskA\ROM.vhd".

Found 16x8-bit Read Only RAM for signal <fib_out>

Summary:

inferred   1 RAM(s).

Unit <ROM> synthesized.

=========================================================================

HDL Synthesis Report

Macro Statistics

# RAMs                                                 : 1

16x8-bit single-port Read Only RAM                    : 1

# Adders/Subtractors                                   : 1

4-bit adder                                           : 1

# Registers                                            : 7

1-bit register                                        : 6

4-bit register                                        : 1

=========================================================================